ALLEN CREATIONS CORP.

Product Specification PS-2000 REV.1A

Durability Test Report

1. MATERIALS

- Contacts: 0.35mm thickness phosphor bronze with 6 u” gold plating, over nickel plated.

- Housing: High Temp. thermoplastic, NYLON UL94V-0, Black

2. ELECTRICAL CHARACTERISTICS

- Relative Humidity: 70%±10% RH

- Operating Temperature: -40℃ to 85℃

- Current Rating: 1.5AAC

- Voltage Rating: 120VAC

3. PERFORMANCE AND TEST DESCRIPTION

The product is designed to meet the electrical, mechanical and environmental Performance requirements specified in Figure1.

4. TEST REQUIREMENTS AND PROCEDURES SUMMARY

| NO | # TEST DESCRIPTION | ## REQUIREMENT | ### TEST RECORD |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1# | 2# | 3# | 4# | 5# |
| 1 | Visual & Mechanical Examination | Mechanical Structure & Appearance & Cosmetic Specs and Drawing. | OK | OK | OK | OK | OK |
|  |  | ### ELECTRICAL |  |  |  |  |  |
| 2 | Termination Resistance | 20mili-ohm Max Initial | 9.32 | 10.2 | 8.63 | 9.32 | 9.31 |
| 3 | Dielectric Withstanding Voltage | No breakage. 1000V is applied between contacts or housing for 1 minute (Cut off current 1mA) | OK | OK | OK | OK | OK |
| 4 | Insulation Resistance | 500M-ohm Min. 500V is applied between contacts or housing for 1 minute. | OK | OK | OK | OK | OK |
|  |  | #### MECHANICAL |  |  |  |  |  |
| 5 | Contact Normal Force | 100g Min. | OK | OK | OK | OK | OK |
| 6 | Durability | Mate and un-mate fo500 cycles at a rate of 20-30 cycles per minute. The termination resistance does not exceed 50mili-ohm. | 20.3 | 19.5 | 18.3 | 20.6 | 19.3 |
| 7 | Mating Force | 8 contacts-------2.3Kgf Max. | 2.02 | 2.10 | 2.10 | 1.95 | 2.10 |
|  |  | #### ENVIRONMENTAL |  |  |  |  |  |
| 8 | Humidity test | At a temperature of 40℃±2℃ and relative Humidity of 90% to 95% for 48 hours. | OK | OK | OK | OK | OK |
| 9 | Dry heat test | Exposing in a heat chamber at a temperature of +60℃±2℃ for 48 hours. | OK | OK | OK | OK | OK |

Figure 1

5. AFTER HUMIDITY TEST & DRY HEAT TEST

| NO | # TEST DESCRIPTION | ## REQUIREMENT | ### TEST RECORD |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1# | 2# | 3# | 4# | 5# |
| 1 | Visual & Mechanical Examination | Mechanical Structure & Appearance & Cosmetic Specs and Drawing. | OK | OK | OK | OK | OK |
|  |  | ### ELECTRICAL |  |  |  |  |  |
| 2 | Termination Resistance | 20mili-ohm Max. Initial | 10.3 | 9.80 | 9.52 | 10.2 | 9.81 |
| 3 | Dielectric Withstanding Voltage | No breakage. 1000V is applied between contacts or housing for 1 minute (Cut off current 1mA) | OK | OK | OK | OK | OK |
| 4 | Insulation Resistance | 500M-ohm Min. 500V is applied between contacts or housing for 1 minute. | OK | OK | OK | OK | OK |
|  |  | #### MECHANICAL |  |  |  |  |  |
| 5 | Contact Normal Force | 100g Min. | OK | OK | OK | OK | OK |
| 6 | Durability | Mate and un-mate for 500 cycles at a rate of 20-30 cycles per minute. The termination resistance does not exceed 50mili-ohm. | 20.5 | 19.9 | 19.3 | 18.6 | 19.9 |
| 7 | Mating Force | 8 contacts-------2.3Kgf Max. | 2.03 | 1.92 | 1.98 | 1.95 | 1.95 |

Approved by: ALLEN YEH                    Inspector: KAO LO